Indianapolis Tracer Data and Meteorological Data

Steven Hanna and Joseph Chang

Earth Tech, 196 Baker Ave., Concord, MA 01742, USA

tel: 508-371-4261   fax: 508-371-4280

e-mail:  shanna@src.com

Helge R. Olesen

National Environmental Research Institute

P.O. Box 358, DK-4000 Roskilde, Denmark

Tel. +45 46 30 12 00, Fax. +45 46 30 11 14

E-mail: hro@dmu.dk

May 1997 (slight edit in October 2005)

The original version of this document was written by Steve Hanna and Joe Chang. The present version has been slightly modified by H.R. Olesen, NERI.

1.	Background

This document briefly describes the Modeling Data Archive (MDA) for the Indianapolis field study.  A full description of the field study is given by TRC (1986) and some results of analysis are given by Hanna and Chang (1991, 1993).  The MDA represents a subset of the full data set, which includes many magnetic tapes full of lidar data and fast-response turbulence data.  The MDA should be sufficient for modelers for evaluation exercises.

The EPRI Indianapolis field study involved SF6 tracer releases from the 83.8 m stack (with diameter 4.72 m) at the Perry K power plant in Indianapolis, Indiana, USA.  The geographic coordinates of this stack are UTM-N 4401.59 km (39.8 latitude) and UTM-E 571.40 km (86.2 longitude).  The elevation of the plant is 214 m. 170 hours of tracer data are available from September and October, 1985, and represent all stability classes and most wind speed ranges.  Data were taken in 8 or 9 hour blocks.  There are a total of 19 such blocks in the Indianapolis dataset.  Meteorological observations were taken from a 94 m height at the top of a building in the middle of the urban area, and from three 10 m towers in urban, suburban, and rural locations (at the urban site the measuring height for some variables was 11 m, however).  Standard National Weather Source (NWS) observations were available from the local airport.  In addition, vertical profiles were taken by minisondes and acoustic sounders.  Concentrations were observed on a network of about 160 ground-level monitors on arcs at distances ranging from 0.25 to 12.0 km from the stack.  Vertical cross-sections of the plume were made by a lidar a few hundred meters downwind of the stack.  The design of the field experiment was similar to earlier EPRI field experiments at the Kincaid and Bull Run power plants.

The 83.8 m stack at the Perry K plant is located in a typical industrial/commercial/urban complex with many buildings within one or two kilometers of the stack.  For example, the Hoosier Dome sports stadium is a few hundred meters to the east.  Our analyses of the data and an independent wind tunnel study have suggested that the Hoosier Dome and the other buildings do not influence the plume, which tended to rise a hundred meters or more above the stack top most of the time.  As a result, our modeling exercises have ignored the effects of nearby individual buildings.  We specify a surface roughness length of 1 m in order to parameterize the overall effect of the buildings on the boundary layer.  We also specify a "minimum Monin-Obukhov length, L" of 50 m during stable conditions for the urban area in order to account for the fact that the urban boundary layer does not stabilize significantly due to the mechanical mixing generated by the buildings and due to the anthropogenic heating in the urban area (Hanna and Chang, 1991).  If other modelers would like to directly model the influence of the buildings or other aspects of the urban area, they can find the buildings' locations in the TRC (1986) report.

The EPRI report "Urban Power Plant Plume Studies", EPRI EA-5468, contains the following description of the urban site:

"The urban site was located at the northwest corner of Ohio and Senate Streets, adjacent to a State employees' parking lot. This area was surrounded by large buildings and received a heavy volume of traffic. The site was located approximately 1.5 km northeast of the Perry-K plant."

2.  Observed Concentration Data Files for Each Monitoring Arc Hour

Concentration data are summarized in the SF6_IND.DAT data file where a separate line of data is given for each monitoring arc for each hour.  The hour notation refers to the one hour period ending at that time.  Explanations of the data entries are given below:

DIST (KM)

The nominal downwind distance to the monitoring arc.  Because it was not possible to site monitors exactly on the arc (buildings and traffic get in the way), the precise distance to a given monitor is likely to be somewhat different than the nominal distance. Twelve distance values are represented in SF6_IND.DAT:   0.25, 0.5, 0.75, 1.0, 1.5, 2, 3, 4, 6, 8, 10, 12 km.

TOTAL SAMPLE

The number of monitors represented at that arc distance for that hour.

NONZERO SAMPLE

The number of monitors that are recording nonzero concentrations at that arc distance for that hour.

ARC MAX (PPT or nG/M3)

The hourly-averaged concentration at the monitor that has the maximum value at that arc distance.  Note PPT = volume parts per trillion, and 1 nG = 10-9 G.

ARCMAX / Q

The normalized arc-wise maximum in units of 10-9 s/m3.

AZIMUTH TO MAX

The azimuth angle in degrees to the monitor registering the ARCMAX (where 0 represents a monitor due east of the stack and angles increase counterclockwise).

CY (μG/M2)

The cross-wind integrated concentration at that arc distance.  It is calculated only if there are at least four monitors registering nonzero concentrations.  The trapezoidal rule is used to perform the integration. Note 1 μG = 10-6 G. (There are 39 hours with a quality index of 3 which have missing values of CY; for these 39 hours the maximum arcwise concentration is reasonably well-defined, but there are only three monitors with non-zero concentrations.)

SIGY (M)

σy, or the standard deviation of the cross-wind concentration distribution at that arc distance.  Like CY, it is calculated only if there are at least four monitors registering nonzero concentrations.  The moment method and the trapezoidal rule are used to calculate σy.

QUALITY INDEX

A subjective numerical ranking of the quality of the data on that monitoring arc.  This ranking was also applied to the Kincaid data distributed previously (in the Model Validation Kit) It has been determined by studying the ground level SF6 observation patterns during each hour and assigning a quality index ranging from 0 to 3 to each monitoring arc.  Monitoring arcs are recommended to be used in the model evaluation exercises only if their quality indicator is 2 or 3.  The definitions for the quality indicators are given below:

Quality

Indicator		Definition

0		This observed maximum concentration should clearly be disregarded (examples:  the plume obviously missed the monitors, the arc has only one or two monitors).

1		This observed maximum concentration is most probably not the maximum value (examples: there are large gaps in the monitoring arc, the observed maximum is isolated in time and space, the maximum is on the edge of the arc).

2		An observed maximum concentration is identified, but the true value may be slightly different (examples: the concentration pattern is irregular, there is no smooth variation from one arc to the next, the maximum is near the edge of the arc).  This category also includes arcs where a "true-zero" concentration is indicated, as suggested by dense coverage of monitors and maxima occurring at that azimuth angle on arcs further downwind.

3		A relatively well-defined maximum concentration is observed, which is continuous in space, is away from the edge of the monitoring arc, and is not irregular or isolated.

3.	Full Concentration Data Files for all Monitors

For each hour, observed concentrations at each receptor are available in files in the subfolder XY_conc. 170 files are provided. The files have names such as

85091611.DAT

(YYMMDDHH.dat)

The file name refers to a date (850916) and an hour (11; LST, end of hour).

An example of data follows:

X           Y         Conc   	Dist

0.129       0.280      7. 	0.310

0.042       0.470    155.  	0.470

0.170       0.440      9.  	0.470

0.228       0.330      0. 	0.400

0.383       0.240      0. 	0.450

X and Y coordinates are indicated in km relative to the source.  (The UTM coordinates for the source are 4401.59 (North, km) and 571.3999 (East, km).)

Conc is observed SF6 concentration (ppt, -999 for missing)

Dist is the source-receptor distance in km.

4.	Observed Meteorology Data Files for Each Hour

Meteorological data for each of the 170 hours are included in the INDI.DAT (in data file which also includes some derived meteorological parameters and stack parameters.  The file is structured so that the columns on the left (through "Perry-K Buoy. Flux") contain data used as input to our HPDM modeling work (Hanna and Chang, 1993), and the columns on the right (beginning with "Rural Wind Dir.") contain data that were not used by us but may be of interest to other modelers. A file with the same contents as INDI.DAT, but in Excel spreadsheet format, is also included (INDI.XLS).

The UTM coordinates for the primary monitoring sites were as follows:

Location	East (km)	North (km)	Elevation (m)

Bank Tower	572.15E	4402.00N	316.2

Urban	571.65E	4402.69N	222.5

Suburban	572.11E	4406.30N	225.0

Rural	564.50E	4401.44N	221.9

Perry K plant, ground	571.40E	4401.59N	214 (approx.)

Several boundary layer parameters in the data file were derived using a surface energy balance method assuming a surface roughness of 1.0 m, an albedo of 0.18, a moisture availability of 0.5, and a minimum Monin-Obukhov length of 50 m (see Hanna and Chang, 1991).

Explanations of the data entries are given below:

YR

Year (e.g., 85)

MO

Month

DY

Day

JULDAY

Julian Day

HR

The hour notation in the file refers to the one-hour period ending at that time. Values range from 1 to 24. The value is Eastern Standard Time, i.e. GMT minus 5 hours.

N_TEST

Number of test (1-19)

SAISTA

SAI Onsite Stab. Class is the so-called Pasquill-Gifford-Turner stability class as determined soon after the experiment by an EPRI contractor (SAI) from the urban wind speeds and the airport cloud cover data.

ZI

Urban Mixing Height (m) is based on our analysis of the urban minisonde profiles.  It was found that urban and rural mixing depths showed little differences.

WD94

WS94

Bank Wind Dir. (degrees) and Speed (m/s) are observed at the 94 m level at the top of the bank building.  These data are used in HPDM to calculate plume rise, plume dilution, and plume trajectory.

SWD94

Bank SIGWD (degrees) is σθ, the standard deviation of the wind direction fluctuation on the bank tower.  When HPDM is run in "observed σθ" mode, this observation is used.

WD11

WS11

T11

Urban Wind Dir. (degrees), Wind Speed (m/s), and Temp (K) are observed at a 11 m height and are used to calculate surface boundary layer parameters u*, H*, L, and w*.

NNWS_

CEILNWS_

PNWS_

National Weather Service Cloud Cover (%), Cloud Height (m), and Pressure (mb) are observed at the NWS airport site and are used in our surface energy balance calculations.

UST

Param. Friction Velocity is u* (m/s), as calculated from observations at the 10 m urban tower and at the airport using surface energy balance considerations.

L

Param. M-L Length is the Monin-Obukhov length, L (m), as calculated from u* and H*.

H_FLUX

Param. Heat Flux is H* (w2/m2) as calculated from observations on the 10 m urban tower and at the airport using surface energy balance considerations.

WST

Param. Conv. Scaling Velocity is the convective scaling velocity, w* (m/s), as calculated from H* and the urban mixing height.

Q

Perry-K Emission Rate (g/s) is the SF6 mass emission rate from the 83.8 m stack.

TQ

Perry-K Exit Temp. (K) is the initial temperature, To, of the plume as it exits the stack.

VSQ

Perry-K Exit Vel. (m/s) is the initial velocity, wo, of the plume as it exits the stack.

BUOY_Q

Perry-K Buoy. Flux (m4/s3) is the initial buoyancy flux, Fo, of the plume.

WDRUR

WSRUR

TRUR

Rural Wind Dir. (degrees), Wind Speed (m/s), and Temp (K) are observed at the 10 m level on the rural tower.

WDSUB

WSSUB

TSUB

Suburban Wind Dir. (degrees), Wind Speed (m/s), and Temp (K) are observed at the 10 m level on the suburban tower.

DIFURB

NETURB

DIRURB

DIFRUR

NETRUR

DIRRUR

Urban and Rural Diffusive Sky Rad., Net Rad., and Direct Solar Rad. (all in w/m2) are components of the radiation budget observed at the urban and rural sites.

There are three "Turbulence groups" with 6 entries for each of three sites: an Urban (index 1), a Rural (index 2) and a Suburban (index 3). The six entries are:

SIGFSTMP

SIGFSTMP is σT the standard deviation (K) of temperature fluctuations. SIGFSTMP(1) is from the urban site, SIGFSTMP(2) from the rural site, and SIGFSTMP(3) from the suburban.

SIGWD

SIGWD is σθ (degrees) at a 10 m height observed by the cup anemometers.

SIGWDSON

SIGWDSON is σθ (degrees) observed by the sonic anemometers.

UWSON

UWSON  isu'w'1 (in m2/s2, equal to -u*2) observed by the sonic anemometer.

WTSON

WTSON isw'T'2 (in K m/s, equal to H*/cρP) observed by the sonic anemometer.

SIGWSON

SIGWSON is σw (m/s) observed by the sonic anemometer.

(end of "Turbulence groups")

The NWS (National Weather Service) group of 4 entries represents observations from the Indianapolis Airport:

STANWS

Stab. Class is the airport Pasquill-Gifford-Turner stability class.

WSNWS_

TNWS_

WDNWS

Wind Speed WSNWS_ (m/s) and Direction WDNWS_ (degrees) are observed at a 6.1 m height at the airport. TNWS_ (K) is NWS temperature.

5.	Other Available Data

The data files provided in this brief report should satisfy most modelers' needs.  If anyone is interested in obtaining the very extensive raw data files from which the above tables were derived, or if they would like to analyze the many magnetic tapes full of lidar data, they should request these data from the authors.  We operate the EPRI Atmospheric Sciences Data Center, which is responsible for archiving and distributing data from all EPRI atmospheric field experiments.  In a case involving satisfying a request for the PERRY-K magnetic tapes or any other effort that would require more than a day of work, the requesting agency may have to negotiate a fee for the time and materials involved.

6.	Points to be noted

There are missing values for a number of variables. Details are listed in the file PAR_INDI.DAT. To most modellers, the missing values will not be any problem, except possibly for the case of September 29, where winds from the 94 m level are missing.

Further, note that there is a mixing height of 0 m for several nighttime hours (September 21, 28 and 29). Rawinsonde showed a ground-based inversion on the hours in question.

References

Hanna, S.R. and J.C. Chang, 1993: Hybrid Plume Dispersion Model (HPDM) improvements and testing at three field sites. Atmos. Environ., 27A, 1491-1508.

Hanna, S.R. and J.C. Chang, 1991: Modification of HPDM for Urban Conditions and Its Evaluation using the Indianapolis Data Set.  Final Report Prepared for EPRI by EARTH TECH, 196 Baker Ave., Concord, MA 10742.

TRC, 1986: Urban power plant plume studies, EPRI Report EA-5468, EPRI, 3412 Hillview Ave, Palo Alto, Ca 94304.